Box to Box copy procedure for the Arion AF2000N

You need to install the channels manually into the MASTER receiver.

This is the receiver that will be copied from.

Place the slave receiver near the Master receiver

Connect a null modem lead between the Master and Slave receiver.

Turn the master receiver on at the mains and leave in STANDBY mode

Leave the power off to the SLAVE receiver for now.

Enter 7 4 6 2 1 on the remote while pointing to the Master receiver.

Turn the mains power ON to the SLAVE receiver.

The transfer will commence.

Once complete disconnect the null modem and watch channels.