Arion AF-2000N Ariter3 Channel Editor Instructions

- From Receiver to PC

Firstly make sure you have a null modem cable connected from your PC to the Arion Satellite Receivers RS232 port

Open up the Ariter3 2000 Model.exe software

Click Receive from STB from the top menu or press the Blue Arrow Icon

Now put your Receiver into standby mode by pressing Standby button

Press 74621 and the LED display on the Arion should now read DATA

You should now see the blue progress bars fill from left to right

Now press OK and then press SAVE AS to save these .ir3 settings

You can now edit channels, transponder, satellites and bouquets/groups

- From PC to Receiver

Once you have edited what you wanted, you can now easily transfer these settings to your Arion AF-2000N Satellite Receiver

Make sure you still have the null modem cable connected from your PC to the Arion Satellite Receivers RS232 port

Open up the Ariter3 2000 Model.exe software if not already open, then choose correct .ir3 file if not already open

Click Send to STB from the top menu or press the Red Arrow Icon

Now unplug the Receiver and then plug the power cord back in

You should now see the blue progress bars fill from left to right

The process will end with the following – click OK and let your Receiver boot up

That is it! You now have now edited your settings in your Arion Receiver